# Quick Start Guide:

# NEXIQ Adapter Wi-Fi Configuration for Android and iOS

# Document Overview

This document describes the basic operation for using the Wi-Fi configuration RP1210 IOCTL calls on Android and iOS. Refer to the appropriate RP1210 API SDK documentation for all other RP1210 information. The document is divided into the following sections:

Wi-Fi Configuration Overview

- Step 1: Get Wi-Fi Config IOCTL 0x20101

- Step 2: Update Wi-Fi Config IOCTL 0x20102

Android Overview

iOS Overview

Sample Xamarin Code

## Wi-Fi Configuration Overview

The Wi-Fi configuration is currently supported on the NEXIQ Blue-Link 2™ and NEXIQ USB-Link™ 3 adapters using Bluetooth Low Energy (BLE). The configuration settings are managed via RP1210 IOCTL calls. Note: Location on the device must be turned on for BLE to function.

The RP1210 IOCTL calls make use of the RP1210_WIFI_DEVICE structures as defined below:

typedef  enum op_mode

{

STATION_MODE = 0,

SOFT_AP_MODE = 1

}RP1210_WIFI_DEVICE_OPERATING_MODE;

struct RP1210WiFiConfigurationData

{

unsigned short key_len;//length of security code

unsigned short essid_len;//length of ssid

char essid[32];

unsigned char  key[128];

};

typedef struct RP1210WiFiConfigurationData RP1210_WIFI_CONFIGURATION_DATA;

struct RP1210WiFiDevice

{

unsigned int operating_mode; //STATION_MODE or SOFT_AP_MODE

unsigned int security; //Enabled/disabled

unsigned int dhcp; //on/off

unsigned int ip_address;

unsigned int subnet;

unsigned int gw;

unsigned int wifi_config_version;

unsigned int configuaration_save_count;

unsigned char wifi_mac_addr[6];

unsigned char bt_mac_addr[6];

unsigned short serial_number_length;

char serial_number_str[16];

char peripheral_name[32];

RP1210_WIFI_CONFIGURATION_DATA wifi_config_data;

};

typedef  struct  RP1210WiFiDevice RP1210_WIFI_DEVICE;

### Step 1: Get Wi-Fi Config IOCTL 0x20101

The RP1210_Ioctl ID 0x20101 command is used for retrieving the Wi-Fi configuration of the adapter. A RP1210_WIFI_DEVICE structure is passed back in the output parameter. Note: The password field is used only when updating Wi-Fi configuration data. It is not transferred when getting the Wi-Fi configuration data.

#define IOCTL_GET_WIFI_CONFIG_DATA 0x20101;

RP1210_WIFI_DEVICE *p_rp1210_wifi_device = new RP1210_WIFI_DEVICE();

strcpy(p_rp1210_wifi_device->peripheral_name, “BL2_001234”);

nClientId = 0

nIoctlId = IOCTL_GET_WIFI_CONFIG_DATA

pInput = NULL

pOutput = (unsigned char*)p_rp1210_wifi_device

RP1210_Ioctl(nClientId, nIoctlId, pInput, pOutput);

### Step 2: Update Wi-Fi Config IOCTL 0x20102

The RP1210_Ioctl ID 0x20102 command is used for setting the Wi-Fi configuration of the adapter. An RP1210_WIFI_DEVICE structure that contains all of the necessary information needs to be provided as the input parameter. For example:

#define IOCTL_UPDATE_WIFI_CONFIG_DATA 0x20102;

RP1210_WIFI_DEVICE *p_rp1210_wifi_device = new RP1210_WIFI_DEVICE();

strcpy(p_rp1210_wifi_device->peripheral_name, "BL2_001234");

p_rp1210_wifi_device->serial_number_length = 6;

strcpy(p_rp1210_wifi_device->serial_number_str, "001234");

p_rp1210_wifi_device->operating_mode = STATION_MODE;

p_rp1210_wifi_device->dhcp = 0;

strcpy(p_rp1210_wifi_device->wifi_config_data.essid, "MySSID");

p_rp1210_wifi_device->wifi_config_data.essid_len = 6;

p_rp1210_wifi_device->security = 1;

p_rp1210_wifi_device->wifi_config_data.key_len = 8;

memcpy(p_rp1210_wifi_device->wifi_config_data.key, "12345678", 8);

p_rp1210_wifi_device->ip_address = 0xC0A80063; // 192.168.0.99

p_rp1210_wifi_device->subnet = 0xffffff00; // 255.255.255.0

p_rp1210_wifi_device->gw =  0xC0A80001; // 192.168.0.1

nClientId = 0

nIoctlId = IOCTL_UPDATE_WIFI_CONFIG_DATA

pInput = (unsigned char*)p_rp1210_wifi_device

pOutput = NULL

RP1210_Ioctl(nClientId, nIoctlId, pInput, pOutput);

If "dhcp" is 1, then the ip_address, subnet, and gw values are ignored. Similarly, if "security" is 0, then the wifi_config_data values are ignored.

## Android Overview

Android requires some additional files and coding in the application for the Wi-Fi configuration support.

The following permissions should be declared in the AndroidManifest.xml:

<uses-permission android:name="android.permission.ACCESS_FINE_LOCATION"/>

<uses-permission android:name="android.permission.ACCESS_COURSE_LOCATION"/>

For Xamarin Android, the provided Nexiq.Rp1210Ble.dll file is necessary for the BLE functionality. The DLL should be added to the Xamarin project as a reference.

A BleWrap object must be created by the app and passed to the DLL as well as the application context be set. The appropriate library must be loaded from the app before calling SetBleWrapObject (“nbl2r32” for NEXIQ Blue-Link 2™ or “nuln3r32” for NEXIQ USB-Link™ 3). For example:

static BleWrap bleWrap = new BleWrap();

Rp1210Application.AppContext = Application.Context;

. . .

switch (model)

{

case “Blue-Link2”:

Java.Lang.JavaSystem.LoadLibrary("nbl2r32");

break;

case “USB-Link3”:

Java.Lang.JavaSystem.LoadLibrary("nuln3r32");

break;

}

Rp1210Application.SetBleWrapObject(bleWrap);

Similarly for Native Android, the provided rp1210ble.jar file can be used to make use of the BLE functionality.

### iOS Overview

There is no special handling or additional files required for iOS. Refer to the sample code in Step 1 and Step 2 for reference.

### Sample Xamarin Code

Following is an example of how to use the RP1210_Ioctl calls in Xamarin.

[StructLayout(LayoutKind.Sequential)]

unsafe public struct RP1210WiFiConfigurationData

{

public ushort key_len;

public ushort essid_len;

public fixed byte essid[32];    // char array

public fixed byte key[128];

};

[StructLayout(LayoutKind.Sequential)]

unsafe public struct Rp1210WifiDevice

{

public uint operating_mode;     // 0 = Station or 1 = AP Mode

public uint security;           // Enabled/disabled

public uint dhcp;               // on/off

public uint ip_address;

public uint subnet;

public uint gw;

public uint wifi_config_version;

public uint configuaration_save_count;

public fixed byte wifi_mac_addr[6];

public fixed byte bt_mac_addr[6];

public ushort serial_number_length;

public fixed byte serial_number_str[16];    // char array

public fixed byte peripheral_name[32];      // char array

public RP1210WiFiConfigurationData wifi_config_data;

}

private unsafe void LoadWifiConfig(IRp1210Api rp1210Api)

{

byte[] output = new byte[512];

string adapterName = “BL2_123456”;

Rp1210WifiDevice configData = new Rp1210WifiDevice();

byte[] temp = Encoding.ASCII.GetBytes(adapterName);

for (int i = 0; i < temp.Length; i++)

{

configData.peripheral_name[i] = temp[i];

}

output = GetBytes(configData);

short ret = RP1210_Ioctl(0, IOCTL_GET_WIFI_CONFIG_DATA, null, output);

if (ret == 0)

{

configData = FromBytes(output);

uint mode = configData.operating_mode;

uint security = configData.security;

string SSID = new string((sbyte*)configData.wifi_config_data.essid);

. . .

}

}

private unsafe void SaveWifiConfig(IRp1210Api rp1210Api)

{

byte[] input = new byte[512];

string adapterName = “BL2_123456”;

string serialNumber = "123456";

string ssid = "MySSID";

Rp1210WifiDevice configData = new Rp1210WifiDevice();

byte[] temp = Encoding.ASCII.GetBytes(adapterName);

for (int i = 0; i < temp.Length; i++)

{

configData.peripheral_name[i] = temp[i];

}

configData.serial_number_length = (ushort)serialNumber.Length;

temp = Encoding.ASCII.GetBytes(serialNumber);

for (int i = 0; i < temp.Length; i++)

{

configData.serial_number_str[i] = temp[i];

}

configData.operating_mode = mode;

configData.wifi_config_data.essid_len = (ushort)ssid.Length;

temp = Encoding.ASCII.GetBytes(ssid);

for (int i = 0; i < temp.Length; i++)

{

configData.wifi_config_data.essid[i] = temp[i];

}

configData.security = 0;

configData.dhcp = 1;

input = GetBytes(configData);

short ret = RP1210_Ioctl(0, IOCTL_UPDATE_WIFI_CONFIG_DATA, input, null);

}

byte[] GetBytes(Rp1210WifiDevice device)

{

int size = Marshal.SizeOf(device);

byte[] arr = new byte[size];

IntPtr ptr = Marshal.AllocHGlobal(size);

Marshal.StructureToPtr(device, ptr, true);

Marshal.Copy(ptr, arr, 0, size);

Marshal.FreeHGlobal(ptr);

return arr;

}

Rp1210WifiDevice FromBytes(byte[] bytes)

{

Rp1210WifiDevice device = new Rp1210WifiDevice();

int size = Marshal.SizeOf(device);

IntPtr ptr = Marshal.AllocHGlobal(size);

Marshal.Copy(bytes, 0, ptr, size);

device = (Rp1210WifiDevice)Marshal.PtrToStructure(ptr, device.GetType());

Marshal.FreeHGlobal(ptr);

return device;

}